Use Device

MB:Asus B85M-G

BIOS version:0940

CPU:i5 4660

GPU:AMD Radeon RX560/HD4600

Audio:Realtek ALC887

Ethernet/WIFI:Realtek 8111G/ BCM4360

Setting BIOS:

CPU:

AdvancedCPU ConfigurationInyel Virtuallzation Technology:Enabled

AdvancedSystem Agent ConfigurationVT-d:Disabled

Use iGPU:

AdvancedSystem Agent ConfigurationPrimary Display:iGPU

AdvancedSystem Agent ConfigurationiGPU Memory:64M

Use GPU card:

AdvancedSystem Agent ConfigurationPrimary Display:PCIE

Use GPU card+iGPU:

AdvancedSystem Agent ConfigurationPrimary Display:PCIE

AdvancedSystem Agent ConfigurationiGPU Memory:64M

AdvancedSystem Agent ConfigurationiGPU Multi-Momltor:Enabled

USB:

AdvancedUSB ConfiguratonLegacy USB Support:Enabled

AdvancedUSB ConfiguratonIntel xHCI Mode:Enabled

AdvancedUSB ConfiguratonEHCI Hand-off:Enabled

Sleep:

AdvancedOnboard Devices ConfigurationSerial Port Configuration Serial Port:Disabled

Boot:

BootFast Boot:Disabled

BootCSMLaunch CSM: Disabled

BootSecure Boot menuOS Type:Other OS

Device status

Sleep:normal

USB Device:normal

Audio:normal,injact alcid=99

HD4600:unusual,Rx560 instead

WIFI:normal

System

OS:MacOS Big Sur11.4

SMBIOS:iMac 15.1

Screenshot